REPORT

Submitted by: HIMANSHI ARORA

Registration Number:12306250

Section: K23UP

Roll no: 37

Date: 29-04-2025

Course Code: CSE 307

Course Title:

Internetworking Essentials

Submitted to: MR. SIMARJIT SINGH MALHI

School of Computer Science and Engineering

Lovely Professional University

Phagwara, Punjab

GITHUB REPOSITORY:

https://github.com/techyhimanshi2023/Internetworkinglab.git

Introduction

HP Infotech, a mid-sized technology enterprise, operates in a five-floor office building, requiring a robust network to support efficient communication and scalability. Each floor hosts multiple devices, including PCs and servers (Email, FTP, HTTP, DNS, DHCP), with specific host requirements: 5551 hosts on Floor 1, 999 on Floor 2, 13000 on Floor 3, 91 on Floor 4, and 5 on Floor 5. This report outlines the network design, including a hybrid topology (mesh for Floors 1–3, star for Floors 4–5), VLSM-based IP addressing, dynamic routing with RIP, and device configurations. The design ensures seamless inter-floor communication, fault tolerance, and support for critical services, implemented in Cisco Packet Tracer.

Network Design Requirements

1. Topology Selection

The network uses a hybrid topology:

- Floors 1–3: A mesh topology connects three routers (R1, R2, R3) for inter-floor communication, ensuring high fault tolerance and redundancy.

- Floors 4–5: A star topology centers around routers (R4, R5), providing simplicity and scalability for smaller floors.

Key Benefits:

- Fault Tolerance: Mesh topology on Floors 1–3 ensures multiple paths for data, reducing downtime.

- Scalability: Star topology on Floors 4–5 allows easy addition of devices.

- Localized Control: Each floor’s star topology isolates intra-floor traffic.

Router Assignments:

- R1: Floor 1 (Email Server)

- R2: Floor 2 (FTP Server)

- R3: Floor 3 (HTTP Server)

- R4: Floor 4 (No Server)

- R5: Floor 5 (DNS and DHCP Servers)

2. VLSM Calculations and IP Addressing Scheme

The network uses VLSM to allocate IP addresses efficiently based on host requirements, using 1.0.0.0/8 (Class A public IPs for Floors 1–3) and 10.0.0.0/8 (Class A private IPs for Floors 4–5). Below are the VLSM calculations and IP assignments for each floor.

2.1 Floor 1 (5551 Hosts)

- Hosts Required: 5553 (5551 PCs + 1 Email Server + 1 Router Interface)

- Subnet Size: 2^13 = 8192 IPs (8190 usable)

- Subnet Mask: /19 (255.255.224.0)

- Network Address: 1.0.0.0

- Usable IP Range: 1.0.0.1–1.0.31.254

- Broadcast Address: 1.0.31.255

- Default Gateway: 1.0.0.1 (R1, FastEthernet0/0)

- Assignments:

  - Gateway: 1.0.0.1

  - Email Server: 1.0.0.2

  - PCs (7 for simulation): 1.0.0.3–1.0.0.9 (static)

2.2 Floor 2 (999 Hosts)

- Hosts Required: 1001 (999 PCs + 1 FTP Server + 1 Router Interface)

- Subnet Size: 2^10 = 1024 IPs (1022 usable)

- Subnet Mask: /22 (255.255.252.0)

- Network Address: 2.0.32.0

- Usable IP Range: 2.0.32.1–1.0.35.254

- Broadcast Address: 2.0.35.255

- Default Gateway: 2.0.32.1 (R2, FastEthernet0/0)

- Assignments:

  - Gateway: 2.0.32.1

  - FTP Server: 2.0.32.2

  - PCs (7 for simulation): 2.0.32.3–2.0.32.9 (static)

2.3 Floor 3 (13000 Hosts)

- Hosts Required: 13002 (13000 PCs + 1 HTTP Server + 1 Router Interface)

- Subnet Size: 2^14 = 16384 IPs (16382 usable)

- Subnet Mask: /18 (255.255.192.0)

- Network Address: 3.0.36.0

- Usable IP Range: 3.0.36.1–3.0.99.254

- Broadcast Address: 3.0.99.255

- Default Gateway: 3.0.36.1 (R3, FastEthernet0/0)

- Assignments:

  - Gateway: 3.0.36.1

  - HTTP Server: 3.0.36.2

  - PCs (7 for simulation): 3.0.36.3–3.0.36.9 (static)

2.4 Floor 4 (91 Hosts)

- Hosts Required: 92 (91 PCs + 1 Router Interface)

- Subnet Size: 2^7 = 128 IPs (126 usable)

- Subnet Mask: /25 (255.255.255.128)

- Network Address: 10.0.0.0

- Usable IP Range: 10.0.0.1–10.0.0.126

- Broadcast Address: 10.0.0.127

- Default Gateway: 10.0.0.1 (R4, FastEthernet0/0)

- Assignments:

  - Gateway: 10.0.0.1

  - PCs (7 for simulation): 10.0.0.2–10.0.0.8 (static)

2.5 Floor 5 (5 Hosts)

- Hosts Required: 8 (5 PCs + 1 DNS Server + 1 DHCP Server + 1 Router Interface)

- Subnet Size: 2^4 = 16 IPs (14 usable)

- Subnet Mask: /28 (255.255.255.240)

- Network Address: 10.0.0.128

- Usable IP Range: 10.0.0.129–10.0.0.142

- Broadcast Address: 10.0.0.143

- Default Gateway: 10.0.0.129 (R5, FastEthernet0/0)

- Assignments:

  - Gateway: 10.0.0.129

  - DNS Server: 10.0.0.130

  - DHCP Server: 10.0.0.131

  - PCs (5): 10.0.0.132–10.0.0.136 (DHCP)

IP Allocation Table

| Floor | Network Address | Usable IP Range | Default Gateway |
| --- | --- | --- | --- |
| Floor 1 | 1.0.0.0/19 | 1.0.0.1–1.0.31.254 | 1.0.0.1 |
| Floor 2 | 1.0.32.0/22 | 1.0.32.1–1.0.35.254 | 1.0.32.1 |
| Floor 3 | 1.0.36.0/18 | 1.0.36.1–1.0.99.254 | 1.0.36.1 |
| Floor 4 | 10.0.0.0/25 | 10.0.0.1–10.0.0.126 | 10.0.0.1 |
| Floor 5 | 10.0.0.128/28 | 10.0.0.129–10.0.0.142 | 10.0.0.129 |

3. Routing Strategy for Inter-Floor Communication & Connectivity

Dynamic Routing Approach (RIP)

A dynamic routing approach using RIP (Routing Information Protocol) is implemented for inter-floor communication, as required by the project, due to:

- Adaptability: RIP automatically updates routes if network changes occur.

- Scalability: Suitable for a multi-router setup with 5 floors.

- Ease of Configuration: Minimal setup compared to other dynamic protocols.

SNAPSHOTS:

OVERALL SCENARIO:

FLOOR 1: E-MAIL SERVER

FLOOR 2: FTP SERVER

FLOOR 3: HTTP SERVER

FLOOR 4: (NO SERVER)

FLOOR5: DNS AND DHCP SERVER

ROUTING:

From 2nd to 3rd 4th and 5th floor

IP ADDRESSING: (COMPUTER & ROUTER)